PSEUDO CODE: DepthToMap click-to-distance pipeline

IMPORT ROS2, OpenCV, NumPy, threading, message types

### CLASS: DepthToMap Node

INIT FUNCTION

- Initialize ROS2 Node named 'depth_to_map_node'

- Create CvBridge for ROS ↔ OpenCV conversions

- Initialize camera intrinsic matrix K as None

- Create a thread lock for shared resources

- Define topic names for depth image, RGB image, camera info

- Initialize variables for depth image, RGB image, clicked point, and shutdown flag

- Initialize a placeholder for display image

- Create threading event for GUI thread control

- Start GUI thread to show images and handle clicks

- Initialize TurtleBot4 navigator

  - If robot is undocked:

    - Log info about docking first

    - Dock the robot to set initial pose

  - Undock robot to be ready for navigation

- Flags for logging shapes only once

- Create ROS subscriptions:

  - CameraInfo → camera_info_callback

  - Depth Image → depth_callback

  - RGB Image (compressed) → rgb_callback

- Start periodic timer every 0.2s → display_images function

### CALLBACK: camera_info_callback(msg)

- Lock shared state

- Extract camera intrinsic matrix K from incoming message

- If not already logged:

  - Log fx, fy, cx, cy

### CALLBACK: depth_callback(msg)

- Try to convert ROS depth image to OpenCV format

- If empty → Log error

- If shape not yet logged → Log shape

- Lock and store depth image

### CALLBACK: rgb_callback(msg)

- Try to decode compressed RGB image to OpenCV format

- If empty → Log error

- If shape not yet logged → Log shape

- Lock and store RGB image

### CALLBACK: mouse_callback(event, x, y)

- If left mouse click:

  - Lock and store clicked pixel coordinate (x,y)

  - Log the clicked pixel

### FUNCTION: display_images()

- Lock and copy current RGB, depth, and clicked point

- If both RGB and depth exist:

  - Copy RGB and depth images for local use

  - Normalize depth to [0,255] range and apply a color map

  - If a click exists:

    - Extract (x,y)

    - Check bounds for RGB and depth

    - Get depth z-value at (x,y) in meters

    - If valid (0.2–5.0m):

      - Annotate RGB with crosshair and circle at click

      - Annotate depth with distance text

  - Horizontally stack RGB and depth display images

  - Lock and store combined display image

### THREAD: gui_loop()

- Create OpenCV window for display

- Set mouse callback for window

- Loop until GUI stop signal set:

  - Lock and copy display image

  - If image exists:

    - Show image in window

    - If 'q' pressed:

      - Log shutdown request

      - Dock the robot for safety

      - Set shutdown flag and GUI stop event

      - Shutdown ROS2

  - Else:

    - Wait for short time

### MAIN FUNCTION

- Initialize ROS2

- Create DepthToMap node

- Create multi-threaded executor

- Add node to executor

- Spin executor to process callbacks and timers

- On KeyboardInterrupt:

  - Signal GUI thread to stop

  - Join GUI thread

  - Destroy node and close OpenCV windows

### NODE: DepthToMap

INPUT TOPICS

- /oakd/rgb/camera_info → CameraInfo → 3×3 intrinsics matrix K

- /oakd/stereo/image_raw → Image → depth image (CV 16-bit)

- /oakd/rgb/image_raw/compressed → CompressedImage → RGB image (JPEG)

OUTPUT

- On-screen GUI window: side-by-side RGB and depth image, with overlayed distance for clicked pixel

- Logs:

  - Intrinsics (fx, fy, cx, cy)

  - Image shapes

  - Clicked pixel coordinates

  - Distance at clicked point (meters)

### MOUSE INPUT

- User clicks on RGB image → (x,y) pixel coordinates stored

PROCESSING

- For each timer cycle:

  - Normalize depth to 0–255 range

  - Colorize depth for display

  - Extract depth z at (x,y) → convert mm → meters

  - If z valid (0.2–5m), overlay distance text & marker

### GUI OUTPUT

- OpenCV window: 'RGB (left) | Depth (right)'

- Shows combined RGB + depth image

- Markers for clicked pixel, with distance annotation

SHUTDOWN

- Press q → dock robot → shutdown ROS → close windows

DATA FLOW

Camera → ROS Topics

/oakd/rgb/camera_info --> K

/oakd/stereo/image_raw --> depth

/oakd/rgb/image_raw/compressed --> RGB

Mouse Click --> (x,y)

Processing:

RGB + depth --> overlay --> GUI Window

(x,y) + depth[y,x] --> z (meters)

Output:

Visual display + log distance